Flash BIOS in EFI shell:

- Ensure the USB drive is in FAT32 format. if not, format it to FAT32.

- Extract the UEFI shell to the root directory of the USB drive, and copy the BIOS flash package to the USB drive.

- Power on and press F7 pop menu to select boot from the USB UEFI shell.

- Select the FS where the USB drive is located. For example, the USB drive is FS0 as shown in the figure below.

- Execute the batch file efiflash.nsh in the BIOS package

- 6.On the first boot after flash BIOS, Please press DEL to enter BIOS Setup and load defaults.

- Flash BIOS in Windows:

- Double-click WinFlash.bat in the BIOS package. Do not perform any other operations during the update. The system will restart automatically after the update is completed.

- 2.On the first boot after flash BIOS, Please press DEL to enter BIOS Setup and load defaults .